SIMATS ENGINEERING
Department of Computer Science & Engineering
ASSESSMENT TOOL 1 – CONSTRAINT-BASED PROBLEM SOLVING

Course Code: CSA12
Course Title: Computer Architecture for Multicore Systems
CO Assessed: CO5
Total Marks: 25

CO5 Focus: Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling, DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt.

# Q1. I/O Communication Mechanism for a Real-Time Embedded System

The system must keep CPU utilization below 15%, handle interrupts generated by multiple sensors every 1 ms, use vectored interrupt handling, and select an I/O technique that provides low latency.

Proposed architecture:

Use memory-mapped I/O (MMIO) for the sensor registers. Each sensor status/data register is assigned a memory address, allowing the CPU to access it using normal load/store instructions.

Connect all sensors to an interrupt controller. Each sensor is assigned a separate interrupt vector or a distinct vector through the controller.

Use vectored interrupts so that, when an interrupt occurs, the processor directly obtains the address of the corresponding interrupt service routine (ISR). This avoids software polling of all sensors.

Keep ISRs very short: acknowledge the interrupt, read the minimum required status/data, store it in a buffer, and defer longer processing to a lower-priority task.

Use interrupt priorities so urgent sensor events are serviced first. If several 1-ms interrupts occur together, the interrupt controller queues or prioritizes them instead of forcing the CPU to poll each device.

Use DMA where sensor data blocks are large enough to justify it, further reducing CPU involvement.

Constraint analysis and justification:

CPU utilization < 15%: achieved by vectored interrupts, short ISRs, buffering, and optional DMA. The CPU performs control and processing rather than continuous polling.

1-ms interrupt period: the interrupt controller must support the required interrupt rate, and the ISR must complete well before the next interrupt period.

Vectored handling: each interrupt source is mapped to an ISR entry, reducing dispatch latency.

MMIO versus I/O-mapped I/O: MMIO is selected because it provides a simple and fast register-access mechanism on modern embedded processors and integrates naturally with the processor's address space. I/O-mapped I/O can be useful on architectures that provide a separate I/O address space, but it is not necessary here.

Result: MMIO + vectored interrupts + prioritized short ISRs provides a low-latency architecture while keeping CPU overhead low.

# Q2. High-Speed I/O Subsystem Using NVMe over PCIe

The storage subsystem requires throughput above 3 GB/s, must use NVMe over PCIe, must keep bulk data transfer away from the CPU, and must use DMA with interrupt notification.

Proposed architecture:

Use an NVMe SSD connected through a PCIe link. PCIe provides high-bandwidth, point-to-point communication between the host and the NVMe controller.

Use the NVMe controller's DMA capability to transfer storage data directly between the SSD and system memory. The CPU does not copy every data block.

The CPU prepares an NVMe command and places it in the appropriate submission queue. The NVMe controller processes the command and performs the data transfer.

After completion, the controller records completion information and generates an interrupt, preferably using MSI/MSI-X where supported.

The interrupt handler processes the completion notification and schedules any required higher-level software processing.

Use sufficiently wide/high-speed PCIe configuration and an NVMe device capable of more than 3 GB/s. The exact PCIe generation and lane count should be selected based on the required effective throughput and system overhead.

Why this satisfies the constraints:

Throughput > 3 GB/s: PCIe-based NVMe is designed for high-speed storage and can provide multi-GB/s throughput when the PCIe link and SSD are appropriately selected.

NVMe over PCIe: directly satisfied by the architecture.

Low CPU involvement: DMA transfers the bulk data directly between the NVMe controller and memory.

Interrupt notification: completion interrupts inform the CPU that an I/O operation has completed instead of requiring continuous polling.

Overall data path: NVMe SSD → PCIe → NVMe controller/DMA → main memory; CPU mainly manages commands and completions.

# Q3. Multi-Device USB and Thunderbolt Communication Architecture

The architecture must support 6 USB peripherals and 2 high-speed Thunderbolt devices, avoid interrupt starvation, use both hardware and software interrupts, and provide fair bus arbitration.

Proposed architecture:

Connect the six USB peripherals through a USB host controller/hub structure. The host controller schedules USB transactions on the shared USB bus.

Connect the two high-speed devices through appropriate Thunderbolt ports/controllers. Thunderbolt provides high-speed links and supports multiple types of traffic.

Use hardware interrupts for urgent events such as USB controller completion/error notifications and high-priority device events.

Use software interrupts or deferred software mechanisms for lower-priority background processing after the immediate hardware event has been acknowledged.

Assign interrupt priorities so one high-rate device cannot indefinitely block other devices. Keep hardware ISRs short and move lengthy processing to deferred tasks.

Use the bus controller's scheduling/arbitration mechanisms to allocate bus service fairly. For competing software requests, use a fair policy such as round-robin or weighted round-robin according to device requirements.

Avoiding interrupt starvation:

Use bounded ISR execution time.

Use interrupt priorities with controlled nesting rather than allowing one source to monopolize the processor.

Coalesce or moderate very frequent interrupts where supported.

Process deferred work using software interrupts/tasks so hardware interrupt service remains short.

Use fair scheduling/arbitration so lower-bandwidth devices continue to receive service.

# Q4. Data Acquisition System at 500 MB/s

The system must continuously acquire data at 500 MB/s, minimize CPU intervention, compare DMA with interrupt-driven I/O, and use memory-mapped I/O for device registers.

DMA transfer versus interrupt-driven I/O:

| Aspect | DMA Transfer | Interrupt-Driven I/O |
| --- | --- | --- |
| Data movement | DMA controller transfers blocks between device and memory. | CPU executes instructions to move/process data after interrupts. |
| CPU involvement | Very low; CPU mainly configures DMA and handles completion. | Higher; frequent interrupts require CPU service. |
| Suitability for 500 MB/s | Highly suitable for continuous high-rate streams. | Poorer for very frequent small transfers because interrupt overhead can become large. |
| Efficiency | Efficient for bulk data. | Efficient mainly for lower-rate or event-oriented transfers. |
| Recommended use | Primary data path for this system. | Use only for control/status or exceptional events. |

Proposed system:

Use MMIO for device control, status, configuration, and DMA-register access.

Configure a DMA controller with source/destination addresses, transfer size, and circular/ring buffers if the stream is continuous.

The acquisition device writes data directly into memory through DMA, avoiding CPU copying.

Use double buffering or a ring buffer so one memory region can be filled while another is processed.

Generate an interrupt after a block or buffer threshold is completed rather than interrupting the CPU for every sample.

The CPU processes completed buffers and monitors errors while the DMA engine continues the high-rate transfer.

Justification: At 500 MB/s, interrupting the CPU for every small data unit would create substantial interrupt and data-copy overhead. DMA provides the required continuous throughput with minimal CPU intervention, while MMIO gives the CPU direct access to device and DMA control registers.

# Q5. Nested, Vectored Interrupt Handling with <10 µs Maximum Latency

The mechanism must support nested interrupts, keep maximum interrupt latency below 10 µs, use vectored interrupts for high-priority devices, and use software interrupts for background tasks.

Proposed interrupt mechanism:

Connect interrupt sources to an interrupt controller capable of priority management and nesting.

Assign high-priority devices dedicated interrupt vectors. On an interrupt, the controller supplies the vector associated with the source so the processor enters the correct ISR quickly.

Allow a high-priority interrupt to pre-empt a lower-priority ISR. Save the necessary processor context before servicing the higher-priority event.

Keep all high-priority ISRs deterministic and short: acknowledge the device, capture essential data, clear the source, and schedule deferred work.

Use software interrupts for background processing such as logging, data formatting, and non-urgent protocol handling.

Use bounded critical sections and avoid disabling interrupts for long periods.

Measure worst-case interrupt latency and ensure vector lookup, context saving, higher-priority ISR entry, and other blocking effects keep the total below 10 µs.

Priority structure:

Highest priority: time-critical hardware devices → vectored hardware ISR.

Medium priority: normal hardware events → vectored or prioritized ISR.

Lower priority: background/deferred work → software interrupt/task.

Latency justification: Vectored interrupts remove the need to poll multiple devices, nesting allows urgent events to pre-empt lower-priority work, and short deterministic ISRs reduce blocking. Together, these techniques support the <10 µs target, provided the selected processor and interrupt controller are fast enough and the implementation is verified by worst-case timing measurement.