SIMATS ENGINEERING
Department of Computer Science & Engineering
ASSESSMENT TOOL 2 – PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

Course Code: CSA12
Course Title: Computer Architecture for Multicore Systems
CO Assessed: CO5
Total Marks: 25

# Q1. DMA-Based Data Transfer Algorithm

The algorithm transfers a block of data from an I/O device to memory using DMA. The CPU only configures the DMA controller and handles the completion interrupt, thereby minimizing CPU involvement.

ALGORITHM DMA_Data_Transfer

INPUT: Device_Address, Memory_Address, Data_Size
OUTPUT: Data transferred to memory

BEGIN
    Disable unnecessary DMA configuration changes

    DMA.Source      ← Device_Address
    DMA.Destination ← Memory_Address
    DMA.Count       ← Data_Size
    DMA.Mode        ← DEVICE_TO_MEMORY
    DMA.Interrupt   ← ENABLE

    Clear_DMA_Status()
    Start_DMA()

    CPU continues other processing

    WHEN DMA_Interrupt occurs:
        status ← Read_DMA_Status()

        IF status = TRANSFER_COMPLETE THEN
            Acknowledge_DMA_Interrupt()
            Set Transfer_Status ← SUCCESS
        ELSE IF status = TRANSFER_ERROR THEN
            Acknowledge_DMA_Interrupt()
            Set Transfer_Status ← ERROR
            Handle_DMA_Error()
        END IF

    RETURN Transfer_Status
END

Explanation: The DMA controller performs the actual data movement. The CPU is not involved in every byte or word transfer. Only a completion or error interrupt requires CPU attention.

# Q2. Priority-Based Vectored Interrupt Handling

The system receives interrupts from multiple devices. High-priority devices must be serviced first, and vectored interrupts allow the processor to directly dispatch the appropriate service routine.

ALGORITHM Interrupt_Handler

INPUT: Interrupt_Request from multiple devices
OUTPUT: Appropriate Interrupt Service Routine (ISR) executed

BEGIN
    WHILE system is running DO

        pending ← Read_Pending_Interrupts()

        IF pending is NOT EMPTY THEN
            selected ← Select_Highest_Priority(pending)

            vector ← Get_Interrupt_Vector(selected)

            Save_CPU_Context()

            IF Priority(selected) > Current_Interrupt_Priority THEN
                Set_Current_Priority(Priority(selected))
                Execute_ISR(vector)
            ELSE
                Queue_Interrupt(selected)
            END IF

            Restore_CPU_Context()
        END IF

    END WHILE
END

FUNCTION Select_Highest_Priority(pending)
BEGIN
    selected ← first interrupt in pending

    FOR each interrupt IN pending DO
        IF Priority(interrupt) > Priority(selected) THEN
            selected ← interrupt
        END IF
    END FOR

    RETURN selected
END

Explanation: The interrupt controller checks pending requests, selects the highest-priority source, obtains its vector, and dispatches the corresponding ISR. Lower-priority requests can remain pending or be queued. This prevents urgent devices from being delayed unnecessarily.

# Q3. Dynamic Selection of Memory-Mapped I/O and I/O-Mapped I/O

The algorithm chooses an I/O technique according to the required latency and bandwidth. The thresholds can be configured for a particular processor.

ALGORITHM Select_IO_Method

INPUT: Required_Latency, Required_Bandwidth
OUTPUT: Selected_IO_Method

CONSTANT LOW_LATENCY_LIMIT
CONSTANT HIGH_BANDWIDTH_LIMIT

BEGIN
    IF Required_Latency <= LOW_LATENCY_LIMIT AND
       Required_Bandwidth >= HIGH_BANDWIDTH_LIMIT THEN

        Selected_IO_Method ← MEMORY_MAPPED_IO

    ELSE IF Required_Latency <= LOW_LATENCY_LIMIT THEN

        Selected_IO_Method ← MEMORY_MAPPED_IO

    ELSE IF Required_Bandwidth >= HIGH_BANDWIDTH_LIMIT THEN

        Selected_IO_Method ← MEMORY_MAPPED_IO

    ELSE

        Selected_IO_Method ← IO_MAPPED_IO

    END IF

    RETURN Selected_IO_Method
END

Explanation: Memory-mapped I/O is selected for demanding low-latency or high-bandwidth register access in this design. I/O-mapped I/O is selected when the requirements are less demanding and the processor provides a separate I/O address space. The thresholds should be tuned to the target architecture.

# Q4. Bus Arbitration for PCIe, USB, and Thunderbolt

The arbitration algorithm manages requests from multiple devices and interfaces. It gives urgent requests appropriate priority while using fair scheduling so that lower-priority devices are not permanently starved.

ALGORITHM Bus_Arbitration

INPUT: Bus_Request_List
OUTPUT: Device selected for bus service

BEGIN
    WHILE Bus is active DO

        requests ← Get_All_Pending_Requests()

        IF requests is EMPTY THEN
            Wait_For_Request()
        ELSE
            high_priority ← Find_Highest_Priority(requests)

            IF high_priority is time_critical THEN
                selected ← high_priority
            ELSE
                selected ← Round_Robin_Select(requests)
            END IF

            Grant_Bus(selected)

            IF selected.Interface = PCIe THEN
                Process_PCIe_Transaction(selected)
            ELSE IF selected.Interface = USB THEN
                Process_USB_Transaction(selected)
            ELSE IF selected.Interface = THUNDERBOLT THEN
                Process_Thunderbolt_Transaction(selected)
            END IF

            Update_Arbitration_State(selected)
            Release_Bus(selected)
        END IF

    END WHILE
END

Explanation: The algorithm first checks pending requests. Time-critical traffic receives priority, while ordinary requests use round-robin selection to improve fairness. The interface-specific transaction is then processed and the arbitration state is updated.

# Q5. Hybrid I/O Communication Mechanism

Low-speed devices are handled by polling because their event rate is small, while high-speed devices use DMA and interrupts to avoid continuous CPU data movement.

ALGORITHM Hybrid_IO_Communication

INPUT: Device_List
OUTPUT: Device data processed

BEGIN
    FOR each device IN Device_List DO

        IF Device_Speed(device) = LOW_SPEED THEN

            status ← Poll_Device_Status(device)

            IF status = DATA_READY THEN
                data ← Read_Device_Data(device)
                Process_Data(data)
            END IF

        ELSE

            Configure_DMA(device)
            Enable_DMA_Interrupt(device)
            Start_DMA(device)

        END IF

    END FOR

    WHEN DMA_Interrupt occurs:
        status ← Read_DMA_Status()

        IF status = TRANSFER_COMPLETE THEN
            Acknowledge_DMA_Interrupt()
            buffer ← Get_Completed_Buffer()
            Process_Data(buffer)
            Prepare_Next_DMA_Transfer()

        ELSE IF status = TRANSFER_ERROR THEN
            Acknowledge_DMA_Interrupt()
            Handle_DMA_Error()
        END IF
END

Explanation: Polling is simple and acceptable for low-speed devices because they do not generate data frequently. For high-speed devices, DMA transfers data directly to memory and an interrupt is generated after a block is transferred. This hybrid approach balances simplicity and CPU efficiency.