1.A program performing signed integer addition using 2’s complement produces

incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation).

Debug the issue by analyzing the binary computation and identifying whether overflow

has occurred. Propose a solution to correctly detect and handle overflow in such

cases.

Program:

#include <iostream>

using namespace std;

int main() {

int8_t a = 120;

int8_t b = 50;

int result = a + b;

// Signed 8-bit overflow check

if (result > 127 || result < -128) {

cout << "Overflow occurred!" << endl;

cout << "Actual result = " << result << endl;

} else {

cout << "Result = " << result << endl;

}

return 0;

}

2. A processor using a ripple carry adder shows significant delay in executing arithmetic

operations in a real-time system. Debug the performance bottleneck by examining how

carry propagates through each bit. Suggest an optimized solution using a carry look-

ahead adder, and explain how it reduces delay.

Program:

#include <iostream>

using namespace std;

// Carry Look-Ahead Adder

int carryLookAheadAdder(int A, int B, int Cin) {

int P[4], G[4], C[5];

C[0] = Cin;

for (int i = 0; i < 4; i++) {

P[i] = ((A >> i) & 1) ^ ((B >> i) & 1);

G[i] = ((A >> i) & 1) & ((B >> i) & 1);

}

// Calculate carries in parallel

C[1] = G[0] | (P[0] & C[0]);

C[2] = G[1] | (P[1] & G[0]) |

(P[1] & P[0] & C[0]);

C[3] = G[2] | (P[2] & G[1]) |

(P[2] & P[1] & G[0]) |

(P[2] & P[1] & P[0] & C[0]);

C[4] = G[3] | (P[3] & G[2]) |

(P[3] & P[2] & G[1]) |

(P[3] & P[2] & P[1] & G[0]) |

(P[3] & P[2] & P[1] & P[0] & C[0]);

int Sum = 0;

for (int i = 0; i < 4; i++) {

int S = P[i] ^ C[i];

Sum |= (S << i);

}

return Sum;

}

int main() {

int A, B;

cout << "Enter two 4-bit numbers: ";

cin >> A >> B;

int result = carryLookAheadAdder(A, B, 0);

cout << "Result = " << result << endl;

return 0;

}

- This eliminates long carry propagation from one bit to the next.

- Therefore, CLA greatly reduces propagation delay and improves processor performance, making it suitable for real-time systems.

3. A multiplication unit implementing Booth’s algorithm gives incorrect results for certain

negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-

pair checking, arithmetic shifts, and sign extension. Identify the possible source of

error and suggest corrections to ensure accurate signed multiplication.

Program:

#include <iostream>

using namespace std;

int main() {

int M = -5;       // Multiplicand

int Q = -3;       // Multiplier

int A = 0;

int Q_1 = 0;

int n = 4;

// Sign-extend multiplicand to 4 bits

M &= 0xF;

Q &= 0xF;

for (int i = 0; i < n; i++) {

int Q0 = Q & 1;

// Check bit pair (Q0, Q-1)

if (Q0 == 1 && Q_1 == 0) {

A = (A - M) & 0xF;       // 10 → Subtract M

}

else if (Q0 == 0 && Q_1 == 1) {

A = (A + M) & 0xF;       // 01 → Add M

}

// Save Q0 before shifting

Q_1 = Q0;

// Arithmetic right shift of A and Q

int sign = A & 0x8;

Q = ((A & 1) << 3) | (Q >> 1);

A = A >> 1;

// Sign extension

if (sign)

A |= 0x8;

}

// Combine A and Q

int result = (A << 4) | Q;

// Convert 8-bit result from 2's complement

if (result & 0x80)

result -= 256;

cout << "Result = " << result << endl;

return 0;

}

4. An embedded system using the restoring division algorithm experiences inefficiency

due to repeated restoration steps, leading to increased execution time. Debug the

algorithm’s workflow and identify where unnecessary operations occur. Propose an

optimized approach using the non-restoring division method, explaining how it

improves performance.

Program:

#include <iostream>

using namespace std;

int main() {

int dividend, divisor;

cout << "Enter dividend: ";

cin >> dividend;

cout << "Enter divisor: ";

cin >> divisor;

int A = 0;

int Q = dividend;

int M = divisor;

int n = 4;

for (int i = 0; i < n; i++) {

// Left shift A and Q

A = (A << 1) | ((Q >> (n - 1)) & 1);

Q = (Q << 1) & 0xF;

// Non-restoring operation

if (A >= 0)

A = A - M;

else

A = A + M;

// Set quotient bit

if (A >= 0)

Q = Q | 1;

}

// Final correction

if (A < 0)

A = A + M;

cout << "Quotient = " << Q << endl;

cout << "Remainder = " << A << endl;

return 0;

}

5. A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

Program:

#include <iostream>

#include <iomanip>

using namespace std;

int main() {

float large = 100000000.0f;

float small = 1.0f;

float result = large + small;

cout << fixed << setprecision(10);

cout << "Large  = " << large << endl;

cout << "Small  = " << small << endl;

cout << "Result = " << result << endl;

return 0;

}